Suggested Specification

SECTION 10 26 13

Wallguard.com Defender Series 2330C.1

## GENERAL

  - SUMMARY

    - This section includes the following wall protection system:

      - Corner Guards

    - Related Sections

      - Section 10 26 16 - Bumper Guards

      - Section 10 26 23 - Protective Wall Covering

      - Section 10 26 33 - Door and Frame Protection

      - Section 09 22 00 - Supports for Plaster and Gypsum Board

  - REFERENCES

    - Abbreviations and Acronyms

      - American Society for Testing and Materials (ASTM)

  - SUBMITTALS

    - General: Submit the following in accordance with Section 01 33 00 “Submittal Procedures”.

    - Product Data

      - Data sheet illustrating product dimensions, options, and related components.

      - Installation instructions.

    - Samples for verification of design suitability and finish.

      - 12” (304) Long sample.

    - Test reports from a qualified independent laboratory showing results in compliance to requirements indicated

  - QUALITY ASSURANCE

    - Manufacturer to have no less than 5 years experience in the production of wall protection products having successful in-service performance.

  - DELIVERY, STORAGE, AND HANDLING

    - Storage: Store wall protection products in original packaging. Protect from weather, extreme temperatures, and moisture.

      - Maintain temperature during storage between 40°- 100°F (4°- 38°C).

      - Store materials flat to prevent twisting or sagging of cartons.

    - Handling: Take adequate measure to prevent damage to materials. Avoid exposure of plastic products to direct sunlight.

  - SITE CONDITIONS

    - Ambient Conditions: Do not install wall protection products until installation areas are enclosed and weatherproof. HVAC system must be operational and maintaining temperature at 65°-75°F (18°-24°C) for at least 72 hours prior to installation.

## PRODUCTS

  - MANUFACTURERS

Wallguard.com

PO Box 1109

Dover Plains, NY 12522

Tel: 877-943-6826

www.wallguard.com

  - DESCRIPTION

    - Model 2330C.1, 2” (51) x 2” (51) x 90° Flush mounted corner guard system with a continuous aluminum retainer and a 1/8” (3) radius snap-on stainless steel cover. Outer edges of retainer shall be tapered, ribbed, and perforated for use with drywall screws, drywall tape, and joint compound. Product is designed for full height, floor to ceiling installation.

  - MATERIALS

    - Stainless Steel: Type 304 (ASTM A240), 22 gauge, with #4 satin finish.

    - Aluminum Retainer: Extruded aluminum, alloy 6063-T6 as described in ASTM B221, nominal 0.070” (1.8) thickness.

## EXECUTION

  - EXAMINATION

    - Examine walls for availability of backing to ensure secure attachment of wall protection products.

    - Verify that HVAC is operating and installation area temperature has been maintained between 65°-75°F (18°-24°C) for at least 72 hours prior to installation.

    - Verify application of wall finishes has been completed in accordance with project finish plan.

  - PREPARATION

    - Surface Preparation: Clean substrate to remove dust and debris.

  - INSTALLATION

    - Acclimate materials to building conditions for at least 24 hours prior to installation.

    - Install wall protection products in accordance with manufacturer’s installation instructions.

  - PROTECTION

    - Protect installed material from damage by other trades. Use materials that will not mark, stain, or leave residue on the product.